✅ Method 1 — Debug directly from QtCreator (best)

1. Build in Debug mode

In QtCreator:

- Select Kit → Debug

- Build → Build Project

Verify:

build type = Debug

2. Configure Run Environment in QtCreator

Open:

Projects → Run → Environment

Add these variables:

Required ROS2 environment

AMENT_PREFIX_PATH=/home/robot/ros2_ws/install
COLCON_PREFIX_PATH=/home/robot/ros2_ws/install
LD_LIBRARY_PATH=/opt/ros/jazzy/lib:/opt/Qt/6.10.2/lib
PYTHONPATH=/opt/ros/jazzy/lib/python3.12/site-packages
ROS_DOMAIN_ID=0

(Adjust paths to your system.)

3. Source ROS2 + workspace automatically

QtCreator does NOT source .bashrc.

Best practice: create a wrapper script.

Create run_l2lidar.sh:

#!/bin/bash
source /opt/ros/jazzy/setup.bash
source ~/ros2_ws/install/setup.bash
exec "$@"

Make executable:

chmod +x run_l2lidar.sh

4. Set QtCreator to use wrapper script

In QtCreator:

Projects → Run

- Executable:

/home/robot/ros2_ws/build/l2lidar_ros2/l2lidar_node

- Run in terminal: ✔️

- Command line arguments:

--ros-args --params-file /home/robot/ros2_ws/src/l2lidar_ros2/config/l2lidar.yaml

- Environment → add:

LD_PRELOAD=/home/robot/run_l2lidar.sh

(or use wrapper as custom executable launcher)

Alternative: set Custom Executable:

/home/robot/run_l2lidar.sh

Arguments:

/home/robot/ros2_ws/build/l2lidar_ros2/l2lidar_node --ros-args --params-file ...

5. Set breakpoints

Good first breakpoints:

- L2LidarNode::L2LidarNode()

- onImuReceived()

- onPointCloudReceived()

- lidar_.ConnectL2()

- processDatagram()

6. Start Debug

Click:

Debug → Start Debugging

You should see:

- ROS2 log output in Application Output

- debugger stops at breakpoints

- UDP packets triggering slots

✅ Method 2 — Debug via ros2 launch (advanced)

If using launch + RViz:

ros2 launch l2lidar_ros2 l2lidar_with_rviz.launch.py

Then attach debugger:

QtCreator:

Debug → Attach to Running Application

Select l2lidar_node.

This is useful later, but harder initially.

🔍 Debugging checklist

Verify in debugger:

- Constructor runs

- ConnectL2() returns true

- imuReceived signal fires

- PCL3DReceived signal fires

- frame.size() > 0

- timestamps valid

- publishers publish

⚠️ Common pitfalls

| Problem | Cause |
| --- | --- |
| Node exits immediately | Qt event loop not running |
| No packets | wrong IP/port |
| No signals | missing Q_OBJECT / moc |
| ROS topics empty | not spinning |
| RViz shows nothing | frame_id mismatch |
| Debugger never hits slots | not using Qt event loop |

🔑 Key rule for your architecture

Because you use Qt signals/slots:

Qt must own the event loop (QCoreApplication::exec), not rclcpp::spin().

Your main should look like:

int main(int argc, char **argv)
{
    rclcpp::init(argc, argv);
    QCoreApplication app(argc, argv);

    auto node = std::make_shared<L2LidarNode>();

    QTimer ros_timer;
    QObject::connect(&ros_timer, &QTimer::timeout, [&]() {
        rclcpp::spin_some(node);
    });
    ros_timer.start(5);

    return app.exec();
}

(Which you already migrated toward.)

🧭 Next debugging strategy (recommended order)

- Debug no RViz, just console logs

- Verify IMU callbacks

- Verify point cloud callbacks

- Echo topics:

ros2 topic echo /imu/data
ros2 topic echo /points

- Then add RViz

If you want, next I can give you a step-by-step QtCreator debugger setup checklist (click-by-click) specifically for your current project layout.
Just say:
“give me QtCreator debugger checklist”

Top of Form

Bottom of Form