Setting up ROS2 on RPI5

For performance reasons one should use a NVMe SSD

To use this you need to add a M.2 HAT for the RPI5 such as:
Geekworm X1001 PCIe to M.2 HAT Key-M NVMe SSD PIP PCIe Peripheral Board

for Raspberry Pi 5 Support M.2 2230/2242/2260/2280 SSD

If you are going to use clangd in develeopment, a 8GB RPI5 may be more suitable than the 4GB.

Install Ubuntu Desktop 24.04 LTS

download

ubuntu-24.04.3-preinstalled-desktop-arm64+raspi.img.xz

flash to NVMe SSD

sudo apt update

sudo apt install curl software-properties-common build-essential -y

sudo curl -sSL https://raw.githubusercontent.com/ros/rosdistro/master/ros.key -o /usr/share/keyrings/ros-archive-keyring.gpg

echo "deb [arch=$(dpkg --print-architecture) signed-by=/usr/share/keyrings/ros-archive-keyring.gpg] http://packages.ros.org/ros2/ubuntu $(lsb_release -cs) main" | sudo tee /etc/apt/sources.list.d/ros2.list > /dev/null

sudo apt update

sudo apt install -y ros-jazzy-desktop

sudo apt install python3-rosdep

sudo rosdep init

sudo rosdep update

rosdep install --from-paths ~/ros2_dashing/ros2-linux/share --ignore-src --rosdistro dashing -y --skip-keys "console_bridge fastcdr fastrtps libopensplice67 libopensplice69 osrf_testing_tools_cpp poco_vendor rmw_connext_cpp rosidl_typesupport_connext_c rosidl_typesupport_connext_cpp rti-connext-dds-5.3.1 tinyxml_vendor tinyxml2_vendor urdfdom urdfdom_headers"

verify

ros2 run demo_nodes_cpp talker

in a new termnial:

ros2 run demo_nodes_cpp listener

other items than need installation:

sudo apt install colcon