Excellent. Below is a complete ROS2 Jazzy + Qt6 C++ repository skeleton named:

l2lidar_ros2

It integrates your existing L2lidar class exactly as specified:

- Uses your Qt6 UDP handling (ConnectL2, DisconnectL2, imuReceived, PCL3DReceived)

- Uses imu() and ConvertL2data2pointcloud(...)

- Supports initialization parameters:

  - Host IP / Port

  - L2 IP / Port

  - Time correction flag

  - Time sync flag

  - Time sync rate (ms)

  - Latency enable

  - IMU adjust

- Publishes:

  - /imu/data (sensor_msgs::msg::Imu)

  - /points (sensor_msgs::msg::PointCloud2)

- Runs Qt event loop + ROS2 spin thread

- Qt6 (not Qt5)

- Jazzy, CMake

- No GUI

📁 Repository Layout

l2lidar_ros2/
├── CMakeLists.txt
├── package.xml
├── launch/
│   └── l2lidar.launch.py
├── params/
│   └── l2lidar.yaml
├── include/l2lidar_ros2/
│   └── l2lidar_node.hpp
└── src/
    ├── l2lidar_node.cpp
    └── main.cpp

Your existing L2lidar sources should be added as a subdirectory or linked library (not duplicated here).

📄 package.xml

<?xml version="1.0"?>
<package format="3">
  <name>l2lidar_ros2</name>
  <version>0.1.0</version>
  <description>ROS2 driver for Unitree L2 LiDAR using Qt6 L2lidar class</description>

  <maintainer email="github@repo">Mark Stegall</maintainer>
  <license>GPL-3.0</license>

  <buildtool_depend>ament_cmake</buildtool_depend>

  <depend>rclcpp</depend>
  <depend>sensor_msgs</depend>
  <depend>std_msgs</depend>

  <build_depend>qt6-base-dev</build_depend>
  <exec_depend>qt6-base-dev</exec_depend>

  <export>
    <build_type>ament_cmake</build_type>
  </export>
</package>

📄 CMakeLists.txt

cmake_minimum_required(VERSION 3.16)
project(l2lidar_ros2)

set(CMAKE_CXX_STANDARD 17)
set(CMAKE_AUTOMOC ON)
set(CMAKE_AUTORCC ON)
set(CMAKE_AUTOUIC ON)

find_package(ament_cmake REQUIRED)
find_package(rclcpp REQUIRED)
find_package(sensor_msgs REQUIRED)
find_package(Qt6 REQUIRED COMPONENTS Core Network)

# ---- Add your L2lidar library here ----
# Example:
# add_subdirectory(external/l2lidar)
# target_link_libraries(l2lidar_ros2_node l2lidar)

include_directories(
  include
  /path/to/your/l2lidar/include
)

add_executable(l2lidar_ros2_node
  src/main.cpp
  src/l2lidar_node.cpp
)

ament_target_dependencies(l2lidar_ros2_node
  rclcpp
  sensor_msgs
)

target_link_libraries(l2lidar_ros2_node
  Qt6::Core
  Qt6::Network
)

install(TARGETS l2lidar_ros2_node
  DESTINATION lib/${PROJECT_NAME}
)

install(DIRECTORY launch params
  DESTINATION share/${PROJECT_NAME}
)

install(DIRECTORY include/
  DESTINATION include/
)

ament_package()

📄 include/l2lidar_ros2/l2lidar_node.hpp

#pragma once

#include <memory>
#include <vector>

#include <QObject>
#include <rclcpp/rclcpp.hpp>
#include <sensor_msgs/msg/imu.hpp>
#include <sensor_msgs/msg/point_cloud2.hpp>

#include "L2lidar.h"   // your existing class
#include "PCpoint.h"

class L2LidarNode : public QObject, public rclcpp::Node
{
    Q_OBJECT

public:
    L2LidarNode();

private slots:
    void onImuReceived();
    void onPcl3DReceived();

private:
    void declareParameters();
    void getParameters();
    void initPublishers();
    void initL2();

    std::unique_ptr<L2lidar> lidar_;

    rclcpp::Publisher<sensor_msgs::msg::Imu>::SharedPtr imu_pub_;
    rclcpp::Publisher<sensor_msgs::msg::PointCloud2>::SharedPtr pcl_pub_;

    std::string frame_id_;
    std::string imu_frame_id_;

    std::string host_ip_;
    std::string lidar_ip_;
    int host_port_;
    int lidar_port_;

    bool enable_time_correction_;
    bool enable_time_sync_;
    uint32_t l2_time_sync_rate_ms_;
    bool enable_latency_;
    bool imu_adjust_;
};

📄 src/l2lidar_node.cpp

#include "l2lidar_ros2/l2lidar_node.hpp"

#include <sensor_msgs/point_cloud2_iterator.hpp>

L2LidarNode::L2LidarNode()
: Node("l2lidar_node")
{
    declareParameters();
    getParameters();
    initPublishers();

    lidar_ = std::make_unique<L2lidar>();

    initL2();
}

void L2LidarNode::declareParameters()
{
    declare_parameter("frame_id", "lidar_link");
    declare_parameter("imu_frame_id", "imu_link");

    declare_parameter("host_ip", "192.168.1.2");
    declare_parameter("host_port", 6201);
    declare_parameter("lidar_ip", "192.168.1.62");
    declare_parameter("lidar_port", 6101);

    declare_parameter("enable_time_correction", true);
    declare_parameter("enable_time_sync", true);
    declare_parameter("l2_time_sync_rate_ms", 1000);

    declare_parameter("enable_latency", true);
    declare_parameter("imu_adjust", true);
}

void L2LidarNode::getParameters()
{
    get_parameter("frame_id", frame_id_);
    get_parameter("imu_frame_id", imu_frame_id_);

    get_parameter("host_ip", host_ip_);
    get_parameter("host_port", host_port_);
    get_parameter("lidar_ip", lidar_ip_);
    get_parameter("lidar_port", lidar_port_);

    get_parameter("enable_time_correction", enable_time_correction_);
    get_parameter("enable_time_sync", enable_time_sync_);
    get_parameter("l2_time_sync_rate_ms", l2_time_sync_rate_ms_);

    get_parameter("enable_latency", enable_latency_);
    get_parameter("imu_adjust", imu_adjust_);
}

void L2LidarNode::initPublishers()
{
    imu_pub_ = create_publisher<sensor_msgs::msg::Imu>("imu/data", 10);
    pcl_pub_ = create_publisher<sensor_msgs::msg::PointCloud2>("points", 10);
}

void L2LidarNode::initL2()
{
    lidar_->LidarSetCmdConfig(
        QString::fromStdString(host_ip_), host_port_,
        QString::fromStdString(lidar_ip_), lidar_port_);

    lidar_->EnableL2TimeCorrection(enable_time_correction_);
    lidar_->EnableL2TSsync(enable_time_sync_);
    lidar_->SetL2TSsyncRate(l2_time_sync_rate_ms_);
    lidar_->EnableLatencyMeasure(enable_latency_);

    QObject::connect(lidar_.get(), &L2lidar::imuReceived,
                     this, &L2LidarNode::onImuReceived);

    QObject::connect(lidar_.get(), &L2lidar::PCL3DReceived,
                     this, &L2LidarNode::onPcl3DReceived);

    if (!lidar_->ConnectL2())
    {
        QString errorstr = lidar_->GetLastUDPError();
        RCLCPP_FATAL(get_logger(), "ConnectL2 failed: %s",
                     errorstr.toStdString().c_str());
        throw std::runtime_error("Failed to connect to L2 lidar");
    }

    RCLCPP_INFO(get_logger(), "L2 lidar connected.");
}

void L2LidarNode::onImuReceived()
{
    auto imu_pkt = lidar_->imu();

    sensor_msgs::msg::Imu msg;
    msg.header.stamp = now();
    msg.header.frame_id = imu_frame_id_;

    msg.orientation.w = imu_pkt.quaternion[0];
    msg.orientation.x = imu_pkt.quaternion[1];
    msg.orientation.y = imu_pkt.quaternion[2];
    msg.orientation.z = imu_pkt.quaternion[3];

    msg.angular_velocity.x = imu_pkt.angular_velocity[0];
    msg.angular_velocity.y = imu_pkt.angular_velocity[1];
    msg.angular_velocity.z = imu_pkt.angular_velocity[2];

    constexpr double G = 9.80665;
    msg.linear_acceleration.x = imu_pkt.linear_acceleration[0] * 0.1 * G;
    msg.linear_acceleration.y = imu_pkt.linear_acceleration[1] * 0.1 * G;
    msg.linear_acceleration.z = imu_pkt.linear_acceleration[2] * 0.1 * G;

    imu_pub_->publish(msg);
}

void L2LidarNode::onPcl3DReceived()
{
    Frame frame;

    if (!lidar_->ConvertL2data2pointcloud(frame, true, imu_adjust_))
        return;

    sensor_msgs::msg::PointCloud2 msg;
    msg.header.stamp = now();
    msg.header.frame_id = frame_id_;

    msg.height = 1;
    msg.width = frame.size();

    sensor_msgs::PointCloud2Modifier modifier(msg);
    modifier.setPointCloud2FieldsByString(2, "xyz", "intensity");
    modifier.resize(frame.size());

    sensor_msgs::PointCloud2Iterator<float> x(msg, "x");
    sensor_msgs::PointCloud2Iterator<float> y(msg, "y");
    sensor_msgs::PointCloud2Iterator<float> z(msg, "z");
    sensor_msgs::PointCloud2Iterator<float> i(msg, "intensity");

    for (const auto &p : frame)
    {
        *x = p.x;
        *y = p.y;
        *z = p.z;
        *i = p.intensity;
        ++x; ++y; ++z; ++i;
    }

    pcl_pub_->publish(msg);
}

📄 src/main.cpp

#include <QCoreApplication>
#include <thread>
#include "l2lidar_ros2/l2lidar_node.hpp"

int main(int argc, char **argv)
{
    QCoreApplication app(argc, argv);
    rclcpp::init(argc, argv);

    auto node = std::make_shared<L2LidarNode>();

    std::thread ros_thread([&](){
        rclcpp::spin(node);
    });

    int ret = app.exec();

    rclcpp::shutdown();
    ros_thread.join();
    return ret;
}

📄 params/l2lidar.yaml

frame_id: "lidar_link"
imu_frame_id: "imu_link"

host_ip: "192.168.1.2"
host_port: 6201
lidar_ip: "192.168.1.62"
lidar_port: 6101

enable_time_correction: true
enable_time_sync: true
l2_time_sync_rate_ms: 1000

enable_latency: true
imu_adjust: true

📄 launch/l2lidar.launch.py

from launch import LaunchDescription
from launch_ros.actions import Node

def generate_launch_description():
    return LaunchDescription([
        Node(
            package="l2lidar_ros2",
            executable="l2lidar_ros2_node",
            name="l2lidar_node",
            parameters=["params/l2lidar.yaml"],
            output="screen"
        )
    ])

✅ This skeleton matches your requirements

✔ Qt6 UDP handled entirely by L2lidar
✔ Uses imuReceived() and PCL3DReceived()
✔ Uses imu() and ConvertL2data2pointcloud()
✔ All initialization parameters implemented
✔ Jazzy compatible
✔ Clean separation of concerns
✔ No GUI
✔ Maintainer = Mark Stegall
✔ CMake only

Top of Form

Bottom of Form