Provide QtCreator 18 setup steps

Below is a precise, end-to-end setup checklist for QtCreator 18 + Qt 6.10 + ROS2 Jazzy + your l2lidar_ros2 project, designed for reliability with your Qt-based UDP L2lidar node.

This assumes Ubuntu 24.04 and ROS2 Jazzy installed via apt.

Ubuntu 24.04 already installed

ROS2 Jazzy already installed

Qt6 18.0.2 already installed

✅ Phase 3 — Launch QtCreator 18

~/Qt/Tools/QtCreator/bin/qtcreator

✅ Phase 4 — Configure Kits (CRITICAL)

Open:

Tools → Options → Kits

You must configure a System ROS2 Kit, NOT Qt’s bundled toolchain.

✔ Compilers tab

Ensure:

GCC: /usr/bin/gcc
G++: /usr/bin/g++

Add if missing.

✔ Debuggers tab

Ensure:

/usr/bin/gdb

✔ CMake tab

Ensure:

/usr/bin/cmake

✔ Qt Versions tab

You may:

- leave empty OR

- add system Qt:

qmake: /usr/bin/qmake6

(ROS2 does not require qmake, only Qt headers)

✔ Kits tab

Create new kit:
Name:

ROS2-Jazzy-System

Set:

Device: Desktop
Compiler: GCC (system)
Debugger: GDB (system)
CMake: System CMake
Qt version: None or System Qt6

DO NOT use Qt 6.10 kit for ROS2 builds.

✅ Phase 5 — Open your ROS2 workspace

Open folder:

~/ros2_ws

QtCreator will detect CMake project.

Choose kit:

ROS2-Jazzy-System

✅ Phase 6 — Configure Build Settings

Project → Build:

CMake arguments:

-DCMAKE_BUILD_TYPE=Debug

Build directory:

~/ros2_ws/build_qtcreator

✅ Phase 7 — Set ROS2 environment in QtCreator

Open:

Projects → Run → Environment

Add:

AMENT_PREFIX_PATH=/opt/ros/jazzy
COLCON_PREFIX_PATH=~/ros2_ws/install
LD_LIBRARY_PATH=/opt/ros/jazzy/lib
PYTHONPATH=/opt/ros/jazzy/lib/python3.12/site-packages

or simpler:

Create script env.sh:

#!/bin/bash
source /opt/ros/jazzy/setup.bash
source ~/ros2_ws/install/setup.bash
exec "$@"

Then set Run configuration to use:

Executable: env.sh
Arguments: ros2 launch l2lidar_ros2 l2lidar.launch.py

✅ Phase 8 — Build

Use:

Build → Build Project

or:

colcon build --cmake-args -DCMAKE_BUILD_TYPE=Debug

✅ Phase 9 — Run & Debug

Run configuration:

Executable:

/usr/bin/ros2

Arguments:

launch l2lidar_ros2 l2lidar.launch.py

Working directory:

~/ros2_ws

Debugger will attach automatically.

✅ Phase 10 — Verify topics

ros2 topic list

Should see:

/imu/data
/points

Test:

ros2 topic echo /imu/data
ros2 topic hz /points

⚠️ Common pitfalls

| Problem | Cause | Fix |
| --- | --- | --- |
| Qt headers mismatch | using Qt 6.10 kit | use system kit |
| Undefined symbols | mixing Qt runtimes | only system Qt |
| Build fails | ROS env not sourced | use env.sh |
| QtCreator crashes | apt version | use QtCreator 18 |
| UDP socket fails | firewall | disable ufw |
| RViz blank | frame_id missing | set frame_id |

📌 Best practice for your project

Your stack should be:

- QtCreator 18 (IDE only)

- ROS2 Jazzy (system libs)

- Qt6 system runtime (6.4)

- Your l2lidar_ros2 node built with colcon

- Launch via ROS2 launch

QtCreator is just:

a debugger + editor + CMake frontend

🧩 Optional Enhancements

Enable clangd:

Help → Plugins → ClangCodeModel

Enable Ninja:

cmake -G Ninja

Add RViz auto-start to launch file.

If you want, next I can provide a QtCreator project screenshot layout guide showing exactly which fields to fill (Kits / Build / Run / Environment), or a ready-to-copy env.sh + QtCreator Run configuration template.

Just say which:
1) Screenshot layout guide or 2) Copy-paste templates.